Sample Tender Document

including possible solutions

Supply of Copy Paper

Tender Reference: KP-2025-01

1. Contracting Authority

Municipality of Musterdorf

Town Hall Square 1, 12345 Musterdorf

2. Type of Procedure and Conract Value

Open procedure in accordance with Directive 2014/24/EU

Below-threshold procurement – no EU-wide publication required

Estimated contract value: approx. €50,000 net

3. CPV-Codes

30197630-1 – Printing and copying paper

30199340-8 – Packaged copying paper

4. Type of contract

Framework contract with binding minimum purchase quantity of 20,000 packs (each pack: 5 × 500 sheets = 2,500 sheets) of white DIN A4 copy paper

Duration: 24 months from award

One-time extension option for a further 12 months

Delivery on call-off

Partial offers, variants and division into lots: not permitted

5. Technical Specifications

| Feature | Requirement |
| --- | --- |
| Product | Copy paper, white, uncoated wood-free |
| Format | DIN A4 (210 × 297 mm) |
| Grammage | 80 g/m² (± 5 %) |
| Recycled content | At least 95 % recovered paper must be used as fibre raw material |
| Bleaching | If bleached, must be Process Chlorine Free (PCF) |
| Paper profile | The paper must have a valid „Paper profile“ |
| Emission limit values (Paper Profile) in kg/t paper | CSB: 4,4 kg/t AOX: 0,07 kg/t SO2: 0,65 kg/t NOx: 1,2 kg/t CO2 fossil: 1.100 kg/t |
| Brightness | ≥ 90 % |
| Thickness | ≥ 100 µm |
| Whiteness CIE | ≥ 150 |
| Opacity | ≥ 90 % |
| Moisture Content | 4,0 – 5,5 % |
| Surface | Smooth, low-dust, suitable for laser, inkjet and copying (single- and double-sided) |
| Packaging | 500 sheets per pack, 5 packs per carton, palletised delivery |
| Delivery time | Max. 10 working days after call-off |
| Delivery locations | Central warehouse + max. 10 additional addresses within the municipality |
| Delivery terms | Delivered duty paid including unloading |

6. Selection criteria

(a) Personal reliability

- Declaration of no grounds for exclusion

- Current commercial register/company register extract (≤ 3 months old)

(b) Economic and financial capacity

- Turnover in paper wholesale at least €50,000/year for the last 3 years

- Public liability insurance with minimum coverage of €500,000

(c) Technical capacity

- At least 3 references of comparable deliveries to public authorities (last 3 years)

- Proof of sufficient storage and logistics capacity

(d) Other declarations and certificates

- Proper payment of taxes and social security contributions

- Environmental management system: The bidder must provide evidence of an environmental management system for the production site of the paper to be supplied under the contract

7. Award criteria

Most economically advantageous tender:

| Award criterion | Weighting |
| --- | --- |
| Price | 100% |
| Response/delivery time Example scoring for delivery time: Delivery within 24 hours of call-off: 20 points Delivery within 48 hours of call-off: 10 points Delivery within 72 hours of call-off: 5 points | 20% |

8. Contract performance conditions

- Minimum call-off quantity: 20 cartons

- Free return/collection of pallets

- Electronic invoicing

- Penalty for delay: 0.5 % per commenced week of delay, maximum 8 % of the call-off value

- Payment: 30 days net

9. Deadline and submission of tenders

- Submission deadline: 15 July 2025, 12:00 noon

- Submission: exclusively electronically via the eSourcing procurement platform

- Bid validity period: at least 60 calendar days

- Language: English

10. Mandatory Annexes to the Tender

- Single European Self-Declaration (ESPd)

- Completed price sheet

- Technical data sheet

- Valid environmental certificate (EU Ecolabel / Blue Angel / Austrian Ecolabel)

- Reference list (minimum 3 references)

- Proof of storage/logistics capacity

- Proof of public liability insurance

- Declaration on compliance with collective agreements and minimum wage (if required)

11. Contact and Legal Remedies

Contact: Ms. Anna Muster

E-Mail: procurement@musterdorf.eu

Legal remedies: Application for review to the competent procurement review authority

Musterdorf, 15th June 2026